Open-source public transportation mobility simulation engine DTALite-S: a discretized space-time network based modeling framework for bridging multi-agent simulation and optimization

Abstract

Recently, an open-source light-weight dynamic traffic assignment (DTA) package, namely DTALite, has been developed to allow a rapid utilization of advanced dynamic traffic analysis capabilities. Aiming to bridge the modeling gaps between multi-agent simulation and optimization in a multi-modal environment, we further design and develop DTALite-S to simplify the traffic flow dynamic representation details in DTALite for future extensions. We hope to offer a unified modeling framework with inherently consistent space-time network representations for both optimization formulation and simulation process. This paper includes three major modeling components: (1) mathematic formulations to describe traffic and public transportation simulation problem on a space-time network; (2) transportation transition dynamics involving multiple agents in the optimization process; (3) an ADMM (Alternating Direction Method of Multipliers) based modeling structure to link different features between multi-agent simulation and optimization used in transportation. This unified framework can be embedded in a Lagrangian relaxation method and a time-oriented sequential simulation procedure to handle many general applications.

Key words

Space-time network, dynamic traffic assignment, multi-agent simulation, Lagrangian relaxation, ADMM (Alternating Direction Method of Multipliers)

# Introduction

To understand and analyze future emerging mobility scenarios, planers and engineers need to utilize many different simulation tools to generate corresponding modeling results. The main purpose of transportation simulation is to shed more light on the underlying mechanisms or potential problems that control the behavior of a complex transportation system.

Typically, simulating a system involves a probabilistic input model, a set of dynamic equations or constraints between the inputs and outputs, and then finally produce a set of outputs under different input instances. Optimization, on the other hand, needs to search an input solution in the dynamic (possibly complex) system subject to a number of constraints. There are a wide range of studies on simulation-based optimization, to name a few, leading research by Osorio et al. [1] involving stochastic urban traffic simulators, and another very recent study by Xiong et al. [2] using the DTALite simulator. Generally, transportation planners and engineers utilize simulation tools to evaluate and further optimize a subset of system’s parameters, but there is a critical modeling gap between simulation and optimization for complex dynamic transportation systems. To bridge such a gap in a multi-modal environment, this research focuses on how to offer a theoretically sound and practically useful modeling framework with a simplified traffic flow dynamic.

## Literature review

Scheduling vehicles on congested transportation networks needs to consider both traffic flows with vehicle-to-road assignment and vehicle routing problem (VRP) with passenger-to-vehicle matching. There are numbers of studies on agent-based traffic assignment and traffic simulation. Mahmassani et al. [3] used flow-density relationships to predict time-dependent traffic flows in Dynamic Network Assignment-Simulation Model for Advanced Roadway Telematics (DYNASMART). From a broader multi-agent optimization perspective, in the study by Nedic et al. [4], a distributed computation model is built for optimizing a sum of convex objective functions for all types of agents. For Shared Autonomous Vehicle (SAV) operating, Fagnant et al. [5] proposed an agent-based shared autonomous vehicle relocation model in order to reduce potential users’ wait times. Following Newell’s kinematic approach (Newell, 1993) [6], Zhou and Taylor [7] designed a mesoscopic traffic simulation approach and developed a time-driven open-source traffic assignment package DTALite to simulate large scale networks with millions of vehicles. Based on multi-source data generated from transportation network companies, Spieser et al. [8] provide an on-demand transportation service model and estimated the benefits of sharing vehicles. Focusing on modeling the microscopic behavior in virtual reality systems, Yu et al. [9] provided a hierarchical modular modeling and distributed simulation methodology. A concise overview of simulation-based transportation analysis approaches is offered by Bierlaire [10].

Transportation researchers have devoted significant attentions on both traffic and public transportation simulation models. Recently, Bradley et al. [11] conducted possible autonomous vehicle (AV) operating scenarios in a road network system, and further modelled the metro transit station as a finite capacity queuing system through a Discrete-Event Simulation (DES) approach, which is also adopted in the study by Afaq et al. [12]. Liang et al. [13] provided a mathematical model to consider the door-to-door intermodal travel trips and found that the train fleet size directly influences the performance of the taxi system. Mahmassani, [14] integrated varying behavioral mechanisms for different classes of vehicles into a microsimulation framework through a series of experiments under varying market penetration rates of AVs and/or connected vehicles. Qu et al. [15] presented a computationally efficient parallel-computing framework for real-life traffic simulation for metropolitan areas. To meet simulation accuracy requirements, Martinez et al. [16] proposed an agent-based model to simulate individual daily mobility while traffic assignment conditions are updated every five minutes. Golubev et al. [17] presented an agent-based traffic modelling framework allowing users to set a specific model for each supported class. Sun et al. [18] presented an agent-based simulation for urban rail transit systems. Based on kinematic wave model, Wen et al. [19] implemented a shared autonomous mobility-on-demand (AMoD) modeling platform for simulating individual travelers and vehicles with demand-supply interaction and analyzing system performance through various metrics of indicators.

From a closely related transportation system optimization perspective, there are many recent papers focusing on vehicle routing optimization models and algorithms used in large-scale optimization. Boyd et al. [20] discussed general distributed optimization and provide efficient implementation under the nonconvex setting. Mahmoudi and Zhou [21] built the state-space-time network to model the vehicle routing problem with pick-up and drop-off and with time windows (VRPPDTW). Based on the Lagrangian decomposition and space-time windows, Tong et al. [22] developed a joint optimization approach for customized bus services. Wei et al. [23] developed a set of integer programming and dynamic programming models to optimize simplified multi-vehicle trajectories. Zhou et al. [24] introduced a vehicle routing optimization engine VRPLite on the basis of a hyper space-time-state network representation with an embedded column generation and Lagrangian relaxation framework.

## Paper structure

The remainder of this paper is organized as follows. The next section represents a modeling framework within a space-time transportation network. In Section 3, we formulate a set of space-time network based formulations describing the public transportation optimization problem. Section 4 provides a simulation process of vehicular loading with pick-up/drop-off services based on Lagrangian relaxation. In Section 5, we use ADMM algorithm to expound the linkage between multi-agent based optimization and simulation. Numerical experiments based on Beijing subway network are conducted in Section 6. In conclusion, Section 7 provides concluding remarks and future research work.

# Problem statement and modeling framework

## 2.1 Motivation

Many disruptive transportation technologies such as autonomous vehicles (AVs) and mobility as a service (MaaS) create new opportunities for public transit to provide high-level services. Along this line, transit operating researchers need to address some theoretical questions, including (a) how to consider individual system dynamics in the context of public transportation network optimization, (b) how to use agent-based simulation tools to evaluate travel schedules with limited road capacity, and (c) how to improve system flexibility and accessibility with a consolidation of modes. To meet these challenges, this paper considers the vehicle route scheduling optimization in the broader framework of public transportation and develops a unified space-time framework to improve public transportation mobility. Table 1 gives an overarching list for different stakeholders and their measures to be examined in our analytical models.

Table 1 Interested stakeholders, and different measures that can be optimized and simulated

| Stakeholders | Interests |  | System-wide metrics | Individual user-oriented performance |
| --- | --- | --- | --- | --- |
| (I) (Non-MaaS) users (passenger/drivers) | - Travel driving cost - Level of service | - Total driving time - Congestion |  | - Waiting time at bottlenecks - Mean travel time - Travel time reliability |
| (II) MaaS passengers | - Travel MaaS service cost - Accessibility | - Total end-to-end service time - MaaS service availability |  | - Waiting time for MaaS vehicles - Service time window and fare - Detour factor for shared ride - Punctuality |
| (III) MaaS operator | - Operating and transportation costs - Revenue and profit from fares - Fleet structure and size of fleet | - Revenue - Ridership - Punctuality - Total travel time |  | - Vehicle traveling distance and travel time - Distance-based average passenger load - Cost per passenger served |
| (IV) Public transportation planning and management agencies | - System efficiency - Public equity - Sustainability | - Space-time accessibility - Motorized traffic - Non-motorized trips |  | - Vehicle traveling distance - MaaS Service rate - Market share of green and shared transportation mode |

Among widely used traffic simulation tools, DTALite is a queue-based mesoscopic traffic simulator, documented in the paper by Zhou and Taylor [7]. It is an open-source mesoscopic DTA (dynamic traffic assignment) simulation package designed to provide transportation planners, engineers, and researchers with a theoretically rigorous and computationally efficient traffic network modeling tool.

The transportation mobility simulation engine DTALite-S to be proposed in this paper is an important extension based on DTALite, with much more consistent representations for optimization in a space-time network. This fully functional, open-source DTALite-S can be downloaded from https://github.com/xzhou99/DTALite-S/tree/master/Version2_PickupDropoffSimulation/DTALite-S. In general, the software suite of DTALite-S aims to:

(1) Provide an open-source simulation package that enables transportation researchers and students to understand the complex space-time network modeling process.

(2) Offer a unified framework with pick-up and drop-off events that cover different traveling activities from driving-only to multiple public transportation modes, across the emerging transportation mobility spectrum, e.g., urban rail transit, synchronized bus, ride-sharing applications as well as freight transport.

(3) Agent-based dynamic traffic assignment and traffic simulation are integrated and extended to tackle practical applications of vehicle routing problem (VRP), or its variants, e.g., vehicle routing problem with pickup and delivery (VRPPD), vehicle routing problem with pickup and delivery with time windows (VRPPDTW).

## 2.2 Overall modelling framework

As shown in Fig. 1, there are 5 different elements in the overall modeling framework. The first category is (1) input data, covering a transportation network, passenger information with origin, destination, preferred departure/arrival time window and desired travel path, vehicle information with depot, time window and capacity constraint. Illustrated in Table 1 with different stakeholders, we consider two different types of vehicles, (I) the background vehicles, driven by their own drivers as Single Occupancy Vehicle (SOV) or High Occupancy Vehicle (HOV) move along the given physical paths from the traffic assignment program, and (III) the MaaS vehicles operated by Maas companies with optimized routes to meet the demand of Maas passengers as stakeholder (II). For the benefits of stakeholder (IV), we have two coupled optimization modules, namely (2) space-time network-based traffic assignment for background vehicles to find the shortest path for each agent, (3) transportation mobility optimization for MaaS vehicles and passengers, which assigns the tasks of serving passengers to specific vehicles in a cost-optimal way. Finally, (4) traffic simulation for calculating traffic states in the transportation network. The system output (5) includes time-dependent travel times, and vehicle and passenger space-time trajectories. The traffic congestion factors, if need to reach the steady states or fixed point equilibrium, should be returned back to part of (1) input data for further consistent optimization and simulation.

Fig. 1 Flow chart with major variables in DTALite-S

# Mathematical programing model in a disseized network

## 3.1. Space-time network construction for a point queue model with constant capacity

Consider a physical transportation network x=(N,L) with a finite set of nodes N and a finite set of links L,  L=Lp∪Lv, where nodes i,j∈N and directed link (i,j)∈L. In this research, the physical transportation network is expanded into two coupled high-dimensional space-time networks, for passengers and vehicles. Beside typical space-time traveling/waiting arcs, additional space-time arcs, such as pick-up and drop-off space-time arc are also be constructed for modeling the vehicle-passenger service process.

A space-time network y=(E,A), where E is the set of space-time vertexes, A=Ap∪Av∪APK is the set of space-time link under planning time horizon T. Each arc (i,j,t,t')∈A indicates a directed space-time path from node i departing at time t to node j arriving at time t'. Thanks to the discretized space-time network structure, it is easy to incorporate passengers’ travel request time windows and vehicles’ operating hour requirement, and more importantly, simplified queueing models (SQM) which can distinguish vehicles’ speed in free-flow and congested conditions, under the assumption of constant bottleneck discharge capacities. The study by Lu et al. [25] provides a more systematic comparison among modes of point queues, spatial queues, and an extended version with time-dependent capacity and spillback along the backward wave. Along this line, one can extend the method from Lawson et al. [26] to calculate the spatial and temporal extents of queue and the actual waiting time spent upstream of a bottleneck. The schematic trajectories of vehicle v from node i to node j in space-time network are shown in Fig. 2.

Fig. 2 Vehicle trajectory from node i to node j with a bottleneck in space-time network

The detailed attributes of vehicles and passengers can be described for the coupled modules of simulation and optimization. Given a set of passengers P and their travel requests, as pick-up/delivery locations op/dp for each passenger p, a set of vehicles V with vehicle capacity Capv and other routing constraints, the simulation problem aims to find a feasible set of vehicle’s space time trajectories. Given an aggregated realization of traffic flow conditions from the simulation module, the optimization module needs to perform matching and provide system-optimal routing instructions between vehicles and passengers. In details, a space-time network can be built systematically using the following steps from Tong et al. [27]. As illustrated in Fig. 3, a vehicle could travel through a sequence of pick-up and drop-off links for different passengers. For a passenger, his first pick-up link along his trip is his own pick-up link, while he can still travel through pick-up/drop-off links for the other passengers. This unique modeling feature is critically important when considering urban rail transit and public transportation bus systems. A station should be represented as a pair of pick-up and drop-off links, as well as a possible dedicated traveling-through link, as 3-5 and 6-8 in Fig. 3.

Fig. 3 A sequence of pick-up and drop-off links

Table 2 Node sequence of passenger and vehicle in their path, the pick-up links for passengers are marked in bold

| Agent | Node sequence |
| --- | --- |
| V1 | depot1-1-2-3-4-5-6-7-8-9-10-depot2 (1-2,4-5,7-8 are pick up links; 3-4,6-7,9-10 are drop off links; 2-3,5-6,8-9 are moving links) |
| P1 | O1-1-2-3-4-5-6-7-D1 (1-2 is his own pick up link; 6-7 is his own drop-off link; 2-3 and 5-6 are moving links, 4-5 is the others’ pick-up link) |
| P2 | O2-1-2-3-4 -D2 (1-2 is pick up link; 3-4 is drop off link; 2-3 is moving link) |
| P3 | O3-7-8-9-10-D3 (7-8 is pick up link; 9-10 is drop off link; 8-9 is moving link) |

Algorithm 1. Build a space-time network

Step 1: Build space-time vertex set E

Add vertex (i,t) to E for i∈N and each t

Step 2: Build space-time transportation arc set Ap/Avfor passengers/vehicles

Step 2.1: Add space-time traveling arc (i,j,t,t+TT(i,j,t)) to Ap/Av, for link (i,j)∈L, where TT(i,j,t) is the link travel time from node i to node j starting at time t

Step 2.2: Add a set of space-time waiting arcs for a pair of vertexes with the same node i, from time t to t+1 Step 3: Add space-time pick-up and drop-off arcs for passengers to consider request time window constraints Step 3.1: Add space-time pick up arc (op',op,t,t+1) to Ap at each time t (EDT(p)≤t<LAT(p)) for each passenger p, where op' is op’s corresponding virtual node Step 3.2: Add space-time drop off arc (dp,dp',t,t+1) to Ap at each time t (EDT(p)<t≤LAT(p)) for each passenger p, where dp' is dp’s corresponding virtual node Step 4: Add space-time origin-departing and destination-returning arcs for vehicles to consider constraint of operating hours Step 3.1: Add space-time departing arc (ov',ov,t,t+1) to Av at each time t (EDT(v)≤t<LAT(v)) for each vehicle v, where ov' is ov’s corresponding origin node Step 3.2: Add space-time arriving arc (dv,dv',t,t+1) to Av at each time t (EDT(v)<t≤LAT(v)) for each vehicle v, where dv' is dv’s corresponding destination node

## 3.2. Mathematic formulations for describing traffic and public transportation optimization problem on a space-time network

The notations used in this section are listed in Table 3, Table 4 and Table 5.

Table 3

Basic indices used to describe the space-time modeling framework

| Index | Definition |
| --- | --- |
| x | Physical transportation network |
| y | Space-time transportation network |
| v | Vehicle index, v ∈V |
| p | Passenger index, p ∈P |
| EDT(p) | Earliest departure time of passenger p |
| LAT(p) | Latest arrival time of passenger p |
| EDT(v) | Earliest departure time of vehicle v |
| LAT(v) | Latest arrival time of vehicle v |
| ov | Origin node of vehicle v, ov∈N |
| op | Origin node of passenger p, op∈N |
| op' | Dummy node for passenger p's origin |
| dv | Destination node of vehicle v, dv∈N |
| dp | Destination node of passenger p, dp∈N |
| dp' | Dummy node for passenger p's destination |
| (i,j,t,t') | Index of space-time traveling arc, i,j,t,t'∈A |
| TTi,j,t | Link travel time from node i to node j starting at time t |

Table 4

Basic parameters used to describe the space-time modeling framework

| Notations | Definition |
| --- | --- |
| TTi,j,t | Link travel time from node i to node j starting at time t |
| Capv | Number of seats in vehicle v, e.g., 4 seats for passenger cars 20-30 seats for a bus, and 300 seats for urban rail train unit |
| CapRoadi,j,t | Road capacity from node i to node j starting at time t, e.g., 1800 vehicles per hour per lane for freeway links, or precisely 1 per time interval for a track in urban rail transit, or multiple buses per time unit depending on the length of the bay in a bus station |
| C(v,i,j,t,t') | Vehicle-specific transportation cost on arc (i,j,t,t') traveled by vehicle v, including transportation costs, vehicle waiting time, converted through drivers’ values of time |
| C(p,i,j,t,t') | Passenger-specific transportation cost on arc (i,j,t,t') traveled by passenger p, including transportation costs, passenger waiting time and converted through passengers’ values of time |

Table 5

Key variables used to describe the space-time modeling framework

| Notations | Definition |
| --- | --- |
| xi,j(p) | Passenger routing variable (=1, if physical arc (i,j) is selected by passenger p; =0, otherwise) |
| xi,j(v) | Vehicle routing variable (=1, if physical arc (i,j) is selected by vehicle v; =0, otherwise) |
| yi,j,t,t'(p) | Passenger space-time routing variable (=1, if space-time arc (i,j,t,t') is selected by passenger p; =0, otherwise) |
| yi,j,t,t'(v) | Vehicle space-time routing variable (=1, if space-time arc (i,j,t,t') is selected by vehicle v; =0, otherwise) |
| z(p,v) | Passenger-to-vehicle marching variable (=1, if passenger p is transported by vehicle v; =0, otherwise), ∀p∈P, v∈V |

Focusing on analytically solvable equations, space-time network-based optimization used in our study need to capture complex and important features including vehicle capacities, pick up, and sharing policies, fare schemes, and hailing strategies such as in-advance and on-demand requests. The optimization problem in DTALite-S can be formulated as a multi-commodity network flow model. The objective function shown in Eq. (1) is to minimize all vehicles and passengers’ total costs.

| Min Z=v∈Vi,j,t,t'∈AvCv,i,j,t,t'×yi,j,t,t'v+p∈P(i,j,t,t')∈ApC(p,i,j,t,t')×yi,j,t,t'(p) | (1) |
| --- | --- |

Flow balance constraints for physical path at vehicle v's node

| j∈N:(i,j)∈Lxi,j(v)-j∈N:(i,j)∈Lxj,i(v)=1, i=ov-1,i=dv0,otherwise xi,jv∈0,1,∀i,j∈Lv,∀v∈V | (2) |
| --- | --- |

Flow balance constraint for space-time path at vehicle v's vertex

| (j,t')∈E:(i,j,t,t')∈Avyi,j,t,t'(v)-j,t'∈E:(i,j,t,t')∈Apyj,i,t',tv =1, i,t=(ov,EDT(v))-1,i,t=(dv,LAT(v))0,otherwise yi,j,t,t'v∈0,1,∀i,j,t,t'∈Av,∀v∈V | (3) |
| --- | --- |

Flow balance constraints for physical path at passenger p's node

| j∈N:(i,j)∈Lxi,j(p)-j∈N:(i,j)∈Lxj,i,(p)=1, i=op-1,i=dp0,otherwise xi,jv∈0,1,∀i,j∈Lp,∀p∈P | (4) |
| --- | --- |

Flow balance constraint for space-time path at vehicle p's vertex

| (j,t')∈E:(i,j,t,t')∈Apyi,j,t,t'(p)-j,t'∈E:(i,j,t,t')∈Apyj,i,t',tp =1, i,t=(op,EDT(p))-1,i,t=(dp,LAT(p))0,otherwise yi,j,t,t'v∈0,1,∀i,j,t,t'∈Ap,∀p∈P | (5) |
| --- | --- |

Consistency constraints between physical path and space-time path for p

| (t,t')∈Tyi,j,t,t'p=xi,jp ∀p∈P,(i,j)∈Lp | (6) |
| --- | --- |

Consistency constraints between physical path and space-time path for v

| (t,t')∈Tyi,j,t,t'v=xi,jv ∀v∈V,(i,j)∈Lv | (7) |
| --- | --- |

Road capacity constraints for all the vehicles using the same space-time arc

| v∈Vyi,j,t,t'(v)≤CapRoadi,j,t, ∀i,j,t,t'∈AV | (8) |
| --- | --- |

Passenger carrying capacity constraints for vehicles

| p∈Pyi,j,t,t'(p)×z(p,v)≤yi,j,t,t'v×Capv, ∀v, i,j,t,t'∈Av | (9) |
| --- | --- |

Matching constraints (10) ensures that each passenger is matched to exactly one vehicle

| v∈Vz(p,v)=1，∀p∈P | (10) |
| --- | --- |

Eqs. (2) to (5) impose passenger /vehicle flow balance constraints in both physical and space-time network. Eqs. (6) and (7) ensure that the path used in the physical network corresponds to the time-index trajectories in the space-time network for each agent. Both classes of MaaS vehicles and passengers use the offline scheduled routes from the passenger-to-vehicle assignment results in the Maas optimization program. In the future, an on-line vehicle-to-passenger matching capability can be a nature extension, for example, following the line of research by Ma et al.[28] Alonso-Mora et al.[29] and Vazifeh et al.[30] Constraint (8) can be viewed as a simplified version of point queue model with constant capacity in a space-time network with waiting arcs, and the related modeling details can be found in Lu et al. [25]. Constraint (9) aims to satisfy the vehicle carrying capacity with combined Eq. (10), which ensures that each passenger is matched to exactly one vehicle. If we need to consider intermodal transfers in the public transportation problem, then one passenger may be served by multiple vehicles in a tour sequence and Eq. (10) needs to be further extended.

# Simulation process of vehicular loading and passenger pick-up and drop-off services

## 4.1 Simulation flowchart based on simple data structure

Illustrated in Algorithm 2, we need to perform two loops of time and agents across different links to check the available road and vehicle carrying capacity. As we follow a point queue-based system, without complicated data structure, we only need to be concerned about very few key variables namely arrival time and departure time of vehicle v on link l: TA(v,l), TDv,l, as well as cumulative arrival/ departure counts of vehicles on link l at time t, A(l,t) and D(l,t).

Algorithm 2. Simulation process in DTALite-S using a simplified queue based model

Step 1: Initialization: prepare input data with given agents’ physical routes, A route could pass through a sequence of transportation, pick-up and drop-off links.

Step 2: Perform simulation

For (t=0; t<T; t ++) // loop for each simulation time t

For (l=0; l< ; l++) // loop for each link in the network

Calculate link capacity CapRoadi,j,t, based on cumulative arrival and departure counts A(l,t-1), D(l,t-1) at previous time interval t-1.

For (v=0; v<V; v ++) // loop for each vehicle in current link l

If (vehicle v is ready to move on link l, i.e. TDv,l=t)

Check available capacity, if CapRoadi,j,t≥1 then move vehicle v and the passengers carried in the vehicle, find the next link l' on its path, TAv,l'=t, TDv,l'=t+FFTT(l') as the next available time to leave from link l'. Update capacity of link l by CapRoadi,j,t-PCE(v), update the cumulative flow counts A(l,t), D(l,t).PCE(v) is the passenger car equivalent value of vehicle v.

For a drop-off link, if passenger and vehicle’s next link is not the same, then the vehicle drops off the passenger and removes it from the current passenger list in the vehicle; and set Capv=Capv+1. For a pick-up link, the vehicle can pick up passengers from the link-based passenger waiting queue, if the vehicle’s carrying capacity Capv is available, and further reduce Capv by Capv-1.

Else vehicle v needs to wait for next time interval, that is, TDv,l=t+1 Endif

End// for loop each vehicle v

End// for loop each link l

End// for loop each time t

Step 3: Output data for statistics collection

In step 3, Illustrated in Fig. 4, the waiting time and queue length can be derived from the grant traffic state variables of A(l,t) and D(l,t). Overall, it could be memory consuming to store a full matrix of variables TA(v,l), TDv,l, one can use dynamically allocated vectors to store and update the link sequences along their paths.

Fig.4 Queue length and travel time various in simulation process, where WT is the waiting time

As illustrated in Fig. 5, the software architecture designed in DTALite-S aims to bridge multi-agent simulation and optimization (integrating passenger-to-vehicle assignment, time-dependent routing) in the open-source DTALite-S modeling package.

Fig. 5 Schematic flow chat of major modeling components of DTALite-S

## 4.2 Further discussions for multi-modal environment

As a discretized space-time network based simulation tool for the multi-modal environment, we need to model various public transportation problems including routing and scheduling of urban rail transit system, bus, taxi with ride-hailing strategies, and with in-advance and on-demand requests. Accordingly, Table 6 shows a detailed comparison of multi-mode agent representation and the related modeling focuses.

Table 6 Application comparison in transportation network

| Transportation mode | Agents | Focuses of modeling |
| --- | --- | --- |
| Urban rail transit | - Passenger oversaturation - Passengers and vehicles have fixed routes | - Waiting time - Walking link - Transfer for xij(p) - Timetable |
| Bus | - yij(v) is not stable/reliable, affect yij(p) | - Waiting time - Walking link - Transfer for xij(p) - Bus service network design for xij(v) - Timetable |
| Taxi | - Real-time passenger-to-vehicle matching | - Affected by traffic congestion, capacity - Dynamically solve assignment of passengers and vehicles - Pick-up and drop-off |
| Driving only | - Vehicles carry passengers | - Traffic assignment problem with user equilibrium (UE) - Focus on yij(v) and xijv, - Vehicle capacity and road capacity - Computational graph |
| Bike | - Passengers carry vehicles | - Routing is decided by passenger |

Finally, as a simplified or “student” version of DTALite on a fully discretized space-time network, DTALite-S has the following distinctions listed in Table 7.

Table 7 Major modeling enhancements from DTALite to DTALite-S

| Index | DTALite | DTALite-S |
| --- | --- | --- |
| Representation scale | Mesoscopic, link based network | Range from mesoscopic link-based and microscopic cell-based network, with additional hyper space-time-state network details for vehicle routing problem with pickup and delivery |
| Traffic dynamics model | Point, spatial and simplified kinematic wave-based queues | Point queue model |
| Agent type | Vehicles | Vehicles and passengers |
| Mode | Driving-only mode | Multiple modes |
| Simulation functionality | Traffic assignment, queue propagation | Traffic assignment, queue propagation, vehicle routing optimization |
| Algorithm | Label correcting algorithm, LR | Dynamic programming, LR, ADMM |
| Time-based simulation | Discrete-time in physical network with fixed/regular time interval (6 seconds) | Discrete-time in space-time network with flexible simulation time interval from 0.1 seconds to 1 minutes |

# An ADMM based framework to understand linkage between multi-agent based optimization and simulation

ADMM is a widely used optimization algorithm and the literature of ADMM can be traced by classical papers by Douglas and Rachford [31] and Boyd et al. [20]. Essentially, it integrates problem decomposition techniques of augmented Lagrangian and block coordination descent.

Consider a space-time network problem with two agents v1and v2. Given v2's trajectory up to time t+h, we need to schedule trajectory of v1 from t to t+h, where h is the optimization time horizon in the future. We can interpret the above problem from perspectives of simulation and optimization.

| Min v∈V(i,j)∈ACij(v)×yij(v) s.t. v∈V(i,j)∈AVyij(v)≤1 | (11) |
| --- | --- |

The standard Lagrangian relaxation form for problem (11) can be re-written as Eq. (12), where λ(i,j) is the vector of Largrangian multiplier for different links.

| Lv,λij=v∈V(i,j)∈ACij(v)×yij(v)+(i,j)∈AVλ(i,j)×(v∈V(i,j)∈AVyij(v)-1)+ | (12) |
| --- | --- |

The ADMM function can be written as Eq. (13), and the algorithm will apply a block coordinate decomposition scheme for each agent v.

| Lv,λp,ρ=v∈V(i,j)∈ACij(v)×yij(v)+(i,j)∈AVλ(i,j)×(v∈V(i,j)∈AVyij(v)-1)++ρ2¦¦(v∈V(i,j)∈AVyij(v)-1)+¦¦ 22 | (13) |
| --- | --- |

Through the record of CapRoadi,j,t in Algorithm 2, the simulation mechanism in DTALite-S first removes the infeasible space-time regions occupied by vehicle v2, to ensure the full feasibility trajectories of v1.  Recall that, CapRoadi,j,t=0 indicates that the space-time resource capacity at link (i,j) at time t has been used by one previously scheduled vehicle, otherwise.

As shown in Table 8, we want to show how the sequential ADMM decomposition can be linked to the vehicle loading process inside simulation. In this context, whether a vehicle can use the resource at i,j,t is dependent on the value of penalty parameters ρ in Eq. (13). Of course, an infinite value of ρ can always lead to feasible solutions, which is equivalent to setting CapRoadi,j,t=0 by removing infeasible space-time solution regions. On the other hand, a small value of ρ could still allow different vehicles to compete for the limited road resources, through an iterative process, depending on their own priority and waits on the objective functions. From the ADMM modeling perspective, without the optimization iteration k, the simulation process can be viewed as a one-shot procedure from time 0 to time T, with the infinite penalty parameters to ensure the full feasibility of space-time agent trajectories.

Table 8 Conceptual algorithmic comparison between ADMM with simulation processes in the space time network modeling

| AMDD as iterative optimization | One-shot simulation |
| --- | --- |
| For (k=0; k<K; k ++) // k is optimization iteration For (v=0; v<V; v ++) // loop for each vehicle in the network For (t=0; t<T; t ++) // loop for each time t Update the cost associated with penalty term ρ Find space-time shortest path for each vehicle v with arc cost Ci,jv, feasibility penalty and Lagrangian multipliers λ(i,j) Endfor//for loop each t Endfor// for loop each vehicle v Update Lagrangian multipliers λ(i,j) Update penalty term ρ, typically from small to large Compute up bound and lower bound When reach termination condition Endfor// for each iteration k | For (t=0; t<T; t ++) // loop for each time t For (v=0; v<V; v ++) // loop for each vehicle v in the network Given previously scheduled trajectories and already used road recourse at different space time slots (i,t) Schedule feasible trajectory from time t to t+h for each vehicle v Mark infeasibility for newly scheduled trajectories Endfor// for loop each vehicle v Endfor// for loop each time t |

Essentially, the ADMM-based iterative optimization procedure can optimize the system-wide costs by iteratively scheduling individual vehicle trajectories with a relatively small ρ and then increasing the value of ρ further aims to enforce the primal feasibility. In contract, the simulation-based process makes scheduling decisions only based on purely local information at current time, and only schedules the trajectories at a short time horizon. Interested readers can further study the use of ADMM in a distributed multi-agent optimization environment, e.g. for distributed automated vehicles, based on recent studies by Nedic and Ozdaglar [4].

# 6. Case study

In this section, we implement the DTALite-S engine to simulate the passenger traveling process in the Beijing subway network as shown in Fig. 6, which contains of 18 urban rail transit lines, 343 stations, and 52 transfer stations.

Fig. 6 Layout of the Beijing subway network

In the proposed modelling framework, the Beijing subway network is represented by a space-time network with 2401 nodes and 3510 links. As shown in Fig.3, an urban rail station can be represented by a set of nodes and arcs. A total of 6,699 train services are modeled into the DTALite-S as vehicle schedules. Up to 1,393,093 passenger transaction record samples on a typical workday during 7:00 to 9:00 are obtained from the AFC system in the Beijing subway network as input passenger agents. To implement the Beijing subway case, the total computing time is about 1,560 seconds. Fig. 7 shows the passenger crowding levels during morning peak hours.

Fig. 7 Passenger crowding in trains over the network

We can also obtain the time-dependent passenger waiting queues at platforms. Fig. 8 shows passenger queue lengths and crowding levels in both directions of Yizhuang line in Beijing.

Fig. 8 Passenger waiting queues and crowding levels in trains on Yizhuang line

To assess a more complete set of simulation performances, we also calculate the absolute deviation value between the simulated arrival time and actual arrival time of each passenger at his/her destination, as shown in Fig. 9. Specifically, the absolute deviations of 65% passengers are within 3 min and the absolute deviations of 82% passengers are within 5 min.

Fig. 9 Passenger waiting queues and crowding levels in trains on the Yizhuang line

# Conclusions

Optimization tends to solve tactical/operational issues while simulation aims to resolve more complex and realistic transportation problems. A numbers of researchers have devoted a great amount of efforts to integrate both optimization approaches and simulation tools to pursue future advanced mobility solutions. For example, stochastic programming and approximate dynamic programming often use simulation to estimate expected values within an optimization framework.

The proposed discrete space-time network based modeling framework can be used to evaluate and further optimize transport strategies from new demand and supply perspectives. It is intended to bridges the modeling gaps for solving complex transportation problems, such as multimodal transportation network design, VRP, urban rail transit scheduling, bus operating and etc. With the embedded Lagrangian relaxation and ADMM algorithm, the framework is able to shed more lights on many fundamental research issues in large-scale dynamic traffic assignment, mesoscopic traffic simulation and vehicle route optimization.

In our future research, a more comprehensive and microscopic cell-based simulation framework will be further studied to simulate more detailed travel decisions, such as passenger route selection and lane changing behavior, car flowing behavior. In addition, we will apply and extend the proposed open-source offline modeling framework to many emerging transportation applications, in a real time and distributed computing environment.

References

[1] Osorio C, Bierlaire M. A Simulation-Based Optimization Framework for Urban Transportation Problems[J]. Operations Research, 2013, 61(6):1333-1345.

[2] Xiong C, Zhu Z, Chen X, et al. Optimal travel information provision strategies: an agent-based approach under uncertainty[J]. Transportmetrica B, 2017(4):1-22.

[3] Mahmassani, H.S., Qin, X., and Zhou, X. (2004). DYNASMART-X Evaluation for Real-Time TMC Application: Irvine Test Bed. US DOT/ FHWATREPS Phase 1.5B Final Report. Maryland Transportation Initiative, University of Maryland.

[4] Nedic A, Ozdaglar A. Distributed Subgradient Methods for Multi-Agent Optimization[J]. IEEE Transactions on Automatic Control, 2009, 54(1):48-61.

[5] Fagnant D, Hall C J, Kockelman K M. The Travel and Environmental Implications of Shared Autonomous Vehicles, Using Agent-Based Model Scenarios[J]. Transportation Research Part C, 2014, 40(1):1-13.

[6] Newell, G. F. (1993). A simplified theory on kinematic waves in highway traffic, part I: general theory. Transportation Research Part B 27(4), 281-287.

[7] Zhou X, Taylor J. DTALite: A queue-based mesoscopic traffic simulator for fast model evaluation and calibration[J]. Cogent Engineering, 2014, 1(1):961345.

[8] Spieser K, Treleaven K B, Zhang R, et al. Toward a Systematic Approach to the Design and Evaluation of Automated Mobility-on-Demand Systems: A Case Study in Singapore[M]// Road Vehicle Automation. Springer International Publishing, 2014.

[9] Yu Y, El Kamel A, Gong G, et al. Multi-agent based modeling and simulation of microscopic traffic in virtual reality system[J]. Simulation Modelling Practice & Theory, 2014, 45(6):62-79.

[10] Bierlaire, Michel. Simulation and optimization: A short review[J]. Transportation Research Part C: Emerging Technologies, 2015, 55:4-13.

[11] Bradley K, Matthew J. R. Fully Autonomous Vehicles: Analyzing Transportation Network Performance and Operating Scenarios in the Greater Toronto area, Canada. TRB 96th Annual Meeting Compendium of Papers.

[12] Afaq K, Jiang Y, et al. Modeling and Simulation of Metro Transit Station Walkway as a State-dependent Queuing System based on the Phase-Type Distribution[J]. International Journal of Traffic and Transportation Engineering, 2016, 5(5):103-111.

[13] Liang X, Gonçalo Homem de Almeida Correia, Arem B V. Optimizing the service area and trip selection of an electric automated taxi system used for the last mile of train trips[J]. Transportation Research Part E Logistics & Transportation Review, 2016, 93:115-129.

[14] Mahmassani H S. 50th Anniversary Invited Article—Autonomous Vehicles and Connected Vehicle Systems: Flow and Operations Considerations[J]. Transportation Science, 2016, 50(4).

[15] Qu Y, Zhou X. Large-scale Dynamic Transportation Network Simulation: A Space-time-event Parallel Computing Approach[J]. Transportation Research Part C, 2017,75:1-16.

[16] Martinez L M, Gonçalo H. A. Correia, José M. Viegas. An agent-based simulation model to assess the impacts of introducing a shared-taxi system: an application to Lisbon (Portugal)[J]. Journal of Advanced Transportation, 2015, 49(3):475-495.

[17] Golubev K, Zagarskikh A, Karsakov A, A Framework for A Multi-agent Traffic Simulation Using Combined Behavioural Models[J]. Procedia Computer Science, 2018, 136:443-452.

[18] Sun Y, Zhang C, Dong K, et al. Multiagent Modelling and Simulation of a Physical Internet Enabled Rail-Road Intermodal Transport System[J]. Urban Rail Transit, 2018.

[19] Wen j, Chen Y, Nassir N, Zhao J, Transit-oriented autonomous vehicle operation with integrated demand-supply interaction[J]. Transportation Research Part C, 2018,97: 216-234.

[20] Boyd S, Parikh N, Chu E. Distributed Optimization and Statistical Learning Via the Alternating Direction Method of Multipliers[J]. Foundations & Trends in Machine Learning, 2010, 3(1):1-122.

[21] Mahmoudi M, Zhou X. Finding Optimal Solutions for Vehicle Routing Problem with Pickup and Delivery Services with Time Windows: A Dynamic Programming Approach Based on State-space-time Network Representations[J]. Transportation Research Part B, 2016, 89(N2,):19-42.

[22] Tong L, Zhou L, Liu J, et al. Customized bus service design for jointly optimizing passenger-to-vehicle assignment and vehicle routing[J]. Transportation Research Part C: Emerging Technologies, 2017, 85:451-475.

[23] Wei Y, Avc C, Liu J, et al. Dynamic programming-based multi-vehicle longitudinal trajectory optimization with simplified car following models[J]. Transportation Research Part B: Methodological, 2017, 106:2.

[24] Zhou X, Tong L, Mahmoudi M, et al. Open-source VRPLite Package for Vehicle Routing with Pickup and Delivery: A Path Finding Engine for Scheduled Transportation Systems[J]. Urban Rail Transit, 2018, 4(2):1-18.

[25] Lu C C, Liu J, Qu Y, et al. Eco-system optimal time-dependent flow assignment in a congested network[J]. Transportation Research Part B: Methodological, 2016, 94:217-239.

[26] Lawson, T. W., Lovell, D. J., and Daganzo, C. F., 1996. Using Input-Output Diagram to Determine Spatial and Temporal Extents of a Queue Upstream of a Bottleneck. Transportation Research Record, No. 1572, pp. 140-147.

[27] Tong L, Zhou X, Miller H J. Transportation network design for maximizing space-time accessibility[J]. Transportation Research Part B: Methodological, 2015, 81:555-576.

[28] Ma, J., Li, X., Zhou, F., & Hao, W. (2017). Designing optimal autonomous vehicle sharing and reservation systems: A linear programming approach. Transportation Research Part C: Emerging Technologies, 84, 124-141.

[29] Alonso-Mora, J., Samaranayake, S., Wallar, A., Frazzoli, E., & Rus, D. (2017). On-demand high-capacity ride-sharing via dynamic trip-vehicle assignment. Proceedings of the National Academy of Sciences, 114(3), 462-467.

[30] Vazifeh, M. M., Santi, P., Resta, G., Strogatz, S. H., & Ratti, C. (2018). Addressing the minimum fleet problem in on-demand urban mobility. Nature, 557(7706), 534.

[31] Douglas J, Rachford H H. On the numerical solution of heat conduction problems in two and three space variables[J]. Transactions of the American Mathematical Society, 1956, 82(2):421-439.